CLASS : XIITH SUBJECT : PHYSICS DATE : DPP NO. : 1

Topic :- Current Electricity

1.   In the electric circuit shown each cell has an emf of 2V and internal resistance of 1Ω. The    external resistance is 2Ω. The value of the current I is(in ampere)

2 Ω

2V ,1 Ω

- 2	         b)	1.25	        c)	0.4	         d)	1.2

2.   A, B, C and D are four resistances of 2Ω, 2Ω, 2Ω and 3Ω respectively. They are used to form a Wheatstone bridge. The resistance D is short circuted with a resistances R in order to get the bridge balanced. The value of R will be

a) 4 Ω	           b)6 Ω	           c)	8 Ω	          d)	3 Ω

3.    The arrangement as shown in figure is called as

Variable P.D.

Total  P.D.

a)	Potential divider	b)	Potential adder

c)	Potential substracter	d)	Potential multiplier

4.     If the balance point is obtained at the 35th cm in a meter bridge, the resistances in the left and   right gaps are in the ratio of

a)	7 : 13	b)	13 : 7	c)	9 : 11	d)	11 : 9

5.   Two electric bulbs rated P1 watt V volts and P2 watt V volts are connected in parallel and V volts    are applied to it. The total power will be

a)	P1+P2watt	         b)	P1P2 watt	         c)	P1P2P1+P2watt	         d)	P1+P2P1P2watt

6.	In a meter bridge a 30Ω resistance is connected in the left gap and a pair of resistances P and Q in the right gap. Measured from the left, the balance point is 37.5 cm, when P and Q are in series and 71.4 cm when they are parallel. The values of P and Q (in ohm) are

a)	40, 10	b)	35, 15	c)	30, 20	d)	25, 25

7.	In an experiment to measure the internal resistance of a cell by potentiometer, it is found that the balance point is at a length of 2m when the cell is shunted by a 5Ω resistance; and is at a length of 3m when the cell is shunted by a 10Ω resistance. The internal resistance of the cell is, then

a)	1.5Ω	b)	10Ω	c)	15Ω	d)	1Ω

8.	Two electroplating cells, one of silver and another of aluminium are connected in series. The ratio of the number of silver atoms to that of aluminium atoms deposited during time t will be

a)	1 : 3	b)	3 : 1	c)	1 : 9	d)	9 : 1

9.	A coil of wire of resistance 50 Ω is embedded in a block of ice and a potential difference of 210 V is applied across it. The amount of ice which melts in 1 sec is

a)	0.262 g	b)	2.62 g	c)	26.2 g	d)	0.0262 g

10.	The resistance of 1 A ammeter is 0.018Ω. To convert it into 10 A ammeter, the shunt resistance required will be

a)	0.18 Ω	b)	0.0018 Ω	c)	0.002 Ω	d)	0.12 Ω

11.	When current flows through a conductor, then the order of drift velocity of electrons will be

a)	1010m/sec	b)	10-2cm/sec	c)	104cm/sec	d)	10-1cm/sec

12.	Which of the following statements is wrong

a)	Voltmeter should have high resistance

b)	Ammeter should have low resistance

c)	Ammeter is placed in parallel across the conductor in a circuit

d)	Voltmeter is placed in parallel across the conductor in a circuit

13.	A material B has twice the specific resistance of A. A circular wire made of B has twice the diameter of a wire made of A. Then for the two wires to have the same resistance, the ratio lB/lA of their respective lengths must be

a)	1	b)	1/2	c)	1/4	d)	2

14.   In the circuit shown below, the power developed in the 6Ω resistor is 6 watt. The power in watts developed in the 4Ω resistor is

a)	16	b)	9	c)	6	d)	4

15.   The value of internal resistance of an ideal cell is

a)	Zero	         b)	0.5 Ω	           c)	1 Ω	                        d)	Infinity

16.   If the electronic charge is 1.6×10-19 C, then the number of electrons passing through a   section of wire per second, when the wire carries a current of 2 A is

a)	1.25×1017  b)	1.6×1017       c) 1.25×1019                              d)	1.6×1019

17.  Two bulbs are working in parallel order. Bulb A is brighter than bulb B. If RA and RB are their   resistance respectively then

a)	RA>RB	b)	RA<RB       c)	RA=RB	        d)	None of these

18.	The amount of chlorine produced per-second through electrolysis in a plate which consumes   100 KW power at 200 V is (Given, electrochemical equivalent of chlorine =0.367×10-3gC-1)

a)	18.35 g 	b) 1.835 g   	c)183.5 g	d)0.1835 g

19.   Three resistors each of 2 ohm are connected together in a triangular shape. The resistance  between any two vertices will be

a)	4/3ohm	 b)   3/4 ohm	           c)	3 ohm	          d)	6 ohm

20.  Two different conductors have same resistance at 0°C. It is found that the resistance of the first conductor at t1℃ is equal to the resistance of the second conductor at t2℃. The ratio of the temperature coefficients of resistance of the conductors,α1α2 is

a)	t1t2	b)	t2-t1t2	c)	t2-t1t1	d)	t2t1